2026 Trisonica troubleshooting Writeup

Noah Hoblet, Rachel Clark, Francis Quinn

Physical side. (Francis)

In an effort to understand some abnormal data, the team met at BSU. It seemed that we were only getting pressure data (that read about 920.3 most of the time) and the other data seemed to be variables that weren't described in the manual all reading 0. We then attempted to use a small fan over the sensor to skew the results; however, the pressure continued to read 920.3, and all other readings remained zero. When we switched to a different but identical Trisonica, we received the same abnormal data, leading me to believe the problem was with the data logger, the wiring or my understanding but probably not the wind sensor.  So, I decided to investigate the home spliced wire we were using to connect the Trisoncia Li-550 to the Anamoment datalogger 422.  The Trisonica's manual describes using a 12-pin connection that terminates all but 6 wires to connect to the data logger. While cross referencing the wiring diagrams from both the Trisonica's and the Anamoment's user manuals, I discovered only 5 wires connected. The yellow wire described in the Trisonica's manual as "Serial transmit data output in EIA232 mode and TX+ data output in EIA422 mode."  was not connected to anything.

Thinking this maybe the issue, I connected the wire. We then attempted to read wind data again and unfortunately continued to receive the same abnormal data. The problem could still be the wiring in my opinion. When I began to unravel the tape on the wiring to access it's connections, much of the heat shrink and plastic holding it all together began to fall apart. I think it's possible that loose wiring or a mismatched wire could be blamed for the abnormal data. Not wanting to needlessly try a bunch of random connections that could damage either the wind sensor or the data dogger we decided a safer option would be to email both Licor who makes the Trisonica and Anamoment who makes the data logger and see if either company had any advice for working with the other. As luck would have it, Licor currently owns Anamoment necessitating only one email.

Software Side (Noah)

I began by trying to approach the Anemometer in the basic ways specified by the LI-550 instructions. I inspected the device to make sure that the cords were connected properly, downloaded the TriSonica app from their website, connected the USB cable to my computer, and tried to open the port within the app to the TriSonica device, only to be met by a crash. I tried this multiple times, all to a similar result. I then tried to change various boxes/settings within the Trisonica app, including to a different port, but was met with failure messages on each of these things.

I then tried to reinstall the app to see if it would have a different result; it didn’t. I also tried to remove and replug in the USB cable, to no real effect. The only difference in Trisonica being that the port would show or not show depending on if it was plugged in. Then I began to examine the port drivers, opening my Windows device manager and going to that port and checking in Windows update for updated port drivers, but discovered they were fully up to date.

At that point, after a discussion with Racine, I downloaded TeraTerm to try to get some data, and I was able to connect the port and get some data, but the data looked nothing like that in the LI-550 instructions, with only the pressure looking as expected, the rest of the fields at 0 and not nearly as many pieces of data as expected.

I began trying to troubleshoot TeraTerm, and discovered that no matter what settings I changed (from the Baud rate, data parity, to the stop rate), and after ensuring saved settings, nothing would either break or fix how the data displayed, which is not what happens in the instructions or on any help resources for the Anemometer with TeraTerm.

I then began to realize that the USB interface was different than that in the LI-550 instructions and attempted to find instructions online to work with the two, to no real result. Then, when the rest of the team arrived, we downloaded the app onto Rachel’s computer to see if it was an issue with mine or Racine’s computer, and had the same result with the TriSonica app after connecting the USB (crashes on attempt to the port), but did notice that the port it displayed for the TriSonica usb was different. We then attempted TeraTerm on Rachel’s computer, once again to the same result of only the pressure data being useful, the formatting weird, and no change to the settings affecting anything.

At that point, we began to wonder if one of the wires from the sensor was broken, or if the sensor itself was broken. As we had two sensors, we tried to use the other sensor and plugged it in, to the exact same result.

Figure 1: