Notes on MEDA Instructions

Noah Hoblet

6/4/2026

MEDA (Mars Environmental Dynamics Analyzer) is made up of six main sensors:

- Wind sensor

- Barometer

- Relative humidity sensor

- Thermocouples (to measure atmospheric temperatures)

- Thermopiles (measure thermal IR brightness of surface and lower atmosphere)

- Radiation and dust sensor (infer particle size distribution

Past rovers have been: Viking, Curiosity, and Insight.

The mission objectives of the Curiosity rover were to:

- Explore Jezero Crate

- Decypher its geological processes and history

- Assess the past habitability of the region

- Assess biosignature preservation. And,

- Search for biosignatures

It is also intended to take samples and prepare them for the trip back to Earth and collect data in preparation for human exploration.

As with all other rover missions, it is designed to progress towards at least one of the three Strategic Knowledge Gaps (SKGs):

- Use technology to enable the production of propellant or consumable oxygen from the Martian atmosphere

- Characterize the dust to better understand its effects on human health

- Measure surface weather to validate global models

MEDA was primarily designed to meet SKG #2, by characterizing the effects of the Martian dust to better understand its effects on human health.

Azimuthally: Measured clockwise from

Air Temperature System (ATS):

- Uses five thermocouple sensors placed around the rover, made up of wires with dissimilar metals, to detect the air temperature

- Its operating range is 73 to 1023 degrees Kelvin, and has a resolution of 0.07 Kelvin. Its response speed is 0.045 seconds

Pressure Sensor:

- Important for understanding global dynamics, dust cycles, etc.

The Martian atmosphere is made up of 95.1% Co2, approximately 30% of which is condensed in the Polar caps annually. Mars, similar to Earth, has Hadley Cell circulation.

Baroclinic Instability: A fluid-dynamic process where the rotation of the planet and temperature gradients cause disturbances to grow

Hydrostatic adjustment: Where fluid adjusts its density to return to equilibrium

- Almost every sensor on the rover is built by Vaisala Incorporated

Barocap: Pressure moves capicator plates. It is not sensitive to the composition of a gas, only the pressure of it, making it behave the same on Mars as on Earth.

It is calibrated for 0-1400 Pascals (Pa). Accurate to plus or minus 5 Pascals.

Uses the condensing power of the air to measure its pressure with electronics. However, temperature is a factor in pressure, and thus must be computed by the device in order to calculate the pressure.

Barocap Resolution: 90% of the confidence range of the pressure valve

Short-term repeatability: Consistency. Measures the baseline

Humidity Sensor:

Works in 203-295 Kelvin at 0% Relative Humidity (RH) or 203-235 Kelvin at 100% Relative Humidity.

It detects relative humidity by absorbing the water vapor in the air, which changes its electrical capacity, which is detected by the sensor.

Thermal Infrared Sensor (TIRS)

- Upward and downward longwave radiation

Five wavelength channels

- Help quantify the surface energy budget (SEB) of Mars

- Determine surface Albedo (reflectiveness)

Mars dust is Lambertian (the radiance is reflected equally)

Upward sensors can be used to indirectly detect water or ice clouds at nighttime.

Allow estimation of heat fluxes.